Digital Watchdog is a leading manufacturer of security and surveillance solutions, offering stunning image quality, advanced hardware capabilities, reliable customer support and lowest total cost of deployment to the analog & IP megapixel surveillance markets. Located in Tampa, FL with manufacturing facilities in Seoul, Korea, Digital Watchdog is committed to delivering powerful security solutions to its customers worldwide.

For additional information, contact:

- DW, Inc., Inc.
	16220 Bloomfield Avenue,

- Cerritos, California 90703 USA

- Phone: +1 888 446-3593

- Web: www.digital-watchdog.com

- E-mail: dw-tech@dwcc.tv

HIGH DEFINITION IP VIDEO FISHEYE CAMERA

- DIVISION 28 – ELECTRONIC SAFETY AND SECURITY

28 20 00 	Video Surveillance

28 21 00 	Surveillance Cameras

28 21 13	IP Cameras

Notes to Specifier:

1.  Where several alternative parameters or specifications exist, or where, the specifier has the option of inserting text, such choices are presented in <bold text>.

2.  Explanatory notes and comments are presented in colored text.

3. CSI MasterFormat 2016 incorporates numerous significant changes affecting electronic safety and security. This document is written to provide flexibility in using either format, although adoption of MasterFormat 2016 is encouraged. The following is a guide to the MasterFormat numbers relevant to the product referenced in this specification.

4. MasterFormat 2014 Specification Category:

28 23 29 - Video Surveillance Remote Devices and Sensors

HIGH DEFINITION IP VIDEO FISHEYE CAMERA

- GENERAL

  - SUMMARY

    - Section includes a 9 MP low light, IR indoor/outdoor IP fisheye camera system with vandal resistant IP-66 rated fisheye housing.

    - Product - A high definition IP video dome camera, based on H.264, H.265 and MJPEG compression, capable of streaming 30 fps at resolutions up to 3000x3000, with dual streaming capability in an indoor/outdoor fisheye housing.

## Related Requirements

      - 27 15 01.13 – Video Surveillance Communications Conductors and Cables

      - 28 05 03 - Safety and Security Network Communications Equipment

      - 28 05 19 – Storage Appliances for Electronic Safety and Security

      - 28 06 20 - Schedules for Video Surveillance

      - 28 23 00 - Video Management System

  - REFERENCES

    - Abbreviations

      - AGC - Automatic Gain Control

      - ARP – Address Resolution Protocol

      - AWB  - Automatic White Balance

      - BLC – Back light compression

      - DHCP - Dynamic Host Configuration Protocol

      - DNR – Digital Noise Reduction

      - DNS - Domain Name Server

      - DDNS – Dynamic Domain Name Server

      - fps - frames per second

      - FTP - File Transfer Protocol

      - GUI – Graphical User Interface

      - HLC – Highlight Compensation

      - HTTP - Hypertext Transfer Protocol

      - ICMP – Internet Control Message Protocol

      - IGMP - Internet Group Management Protocol

      - IP - Internet Protocol

      - JPEG - Joint Photographic Experts Group

      - MJPEG - Motion JPEG

      - NTP - Network Time Protocol

      - PoE - Power over Ethernet

      - QoS – Quality of Service

      - RARP – Reverse Address Resolution Protocol

      - RTP - Real-Time Transport Protocol

      - RTSP - Real-Time Streaming Protocol

      - SMTP - Simple Mail Transfer Protocol

      - TCP - Transmission Control Protocol

      - UDP - User Datagram Protocol

      - VMS - Video Management System

      - WDR – Wide Dynamic Range

    - Reference Standards

      - Network

        - IEEE - 802.3 Ethernet Standards

      - Video

        - ISO / IEC 14496 – MPEG-4

          - ISO / IEC 14496–10, MPEG-4 Part 10 (ITU H.264)

        - ISO / IEC 10918 – JPEG

        - ONVIF – Profile S

      - Emissions

        - FCC-47 CFR Part 15 Class B

      - Environmental

        - ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

        - International Electrotechnical Commission (IEC) – Ingress Protection Rating IP66

  - SUBMITTALS

    - Product Data

      - Manufacturer’s printed or electronic data sheets

      - Manufacturer’s installation and operation manuals

      - Warranty documentation

  - QUALIFICATIONS

    - Manufacturer shall have a minimum of five years’ experience in producing IP video equipment.

    - Installers shall be trained and authorized by the Manufacturer to install, integrate, test, and commission the system.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver the camera in the manufacturer’s original, unopened, undamaged container with identification labels intact.

    - Store the camera in a temperature environment of -4° F to 122° F (-20° F to 50° F), protected from mechanical and environmental conditions as designated by the manufacturer.

  - WARRANTY AND SUPPORT

    - Manufacturer shall provide a limited 5-year warranty for the product to be free of defects in material and workmanship.

END OF SECTION

- PRODUCTS

  - EQUIPMENT

    - Manufacturer: 	Digital Watchdog, Inc.

16220 Bloomfield Avenue. Cerritos,

California USA 90703 USA

Phone: (866) 446-3595

Web: www.digital-watchdog.com

E-mail:  dw-tech@dwcc.tv

    - Models DWC-PVF9M2TIR

    - Alternates: None

  - GENERAL DESCRIPTION

    - The 5 MP low light, IR indoor/outdoor IP fisheye camera system (“IP camera”) shall provide video performance capable of providing selectable resolutions up to 3000x3000 pixels at 30 frames per second (fps) and contained within an IP-66 rated fisheye dome housing.

    - Image dewarping makes the fisheye output easy to view and scan for intruders.

    - Embedded PTZ control within the fisheye video allows users to control the part of the 360° view to focus on

    - The IP camera shall possess the following characteristics:

      - H.264, H.265 and MJPEG compression

      - 9 megapixels

      - two independent IP video streams (dual streaming)

      - day/night operation with IR cut filter

      - option for Integral IR LED illuminator providing 55-foot distance

      - Two way audio

      - integral motion detection

      - 3D digital noise reduction

      - PoE capable

      - multicast or unicast capable

      - local storage via Micro SD or SDHC card

      - built-in web server

      - Dynamic DNS (DDNS) support

      - IP-66 rated

  - VIDEO

    - Imager

      - Sensor:	 1/1.7" 9MP Sony CMOS

      - Minimum illumination

        - Color mode: 		0.2 lux (F2.2)

        - Black & white mode: 	0 lux (F2.2)

      - Scanning - Progressive

      - Image Control Settings shall be available for:

        - Automatic white balance (AWB)

          - Selectable for Auto or Manual

            - Manual mode to provide the ability to configure red offset, blue offset and warmth or coolness of light.

        - Exposure modes: 		automatic or manual

          - In manual exposure mode, configurable settings for:

            - brightness

            - shutter mode: 	automatic, anti-flickering or manual – 1/30 to 1/12,000

            - digital slow shutter to decrease shutter speed in low light

            - Automatic Gain Control (AGC)

            - Wide Dynamic Range (WDR) level

        - Day and night settings, to allow configuration for Day (color), Night (Black and White), or Automatic.

        - Frequency

        - Image sharpness

        - <IR LED control>

        - Chroma

        - Image mirror or flip

        - Digital noise reduction

      - Lens

        - Lens type: 2.1mm Fisheye lens

        - Field of view (FOV): 360o

        - Double shutter Wide Dynamic Range

        - Shutter speed:  1/30 sec to 1/12,000

    - Video Streams

      - The IP camera shall support two configurable video streams, each of which may have the following properties:

          - Compression Type:

            - H.264

            - H.265

            - MJPEG

          - Resolution:

            - 3000p at 30fps

        - Frame rate:	0 – 30 fps

    - Motion Detection – The IP camera shall have the ability to detect motion within user defined areas of the video image.

      - Configuration settings shall be available for sensitivity and dwell time.

    - Storage and Recording

      - The IP camera shall have onboard SD card storage.

        - Card type:	Micro SD/SDHC/SDXC Class 10

      - Local recording on the SD card shall commence upon loss of network connectivity, based on a pre-programmed schedule.

      - The local SD storage shall have the ability to be backed up to alternate media without removal of the SD card from the camera.

    - ONVIF – Video streams shall be capable of supporting ONVIF protocol, profile S.

    - Fisheye Views

    - The 5 MP low light, IR indoor/outdoor IP fisheye camera system shall include the following four (4) display options to allow the user to display the 360° video in the best mode for any surveillance scenario:

      - Original 360° Image display mode displays the entire view of the 360° image in a round image layout.

      - Single Panorama Image display mode displays the entire view of the 360° image in a flattened panoramic layout.

      - Double Panorama Image display mode splits the 360° image into two (2) horizontal 180° flattened panoramic layouts.

      - Four Image display modes display a single 90° inset view from within the 360° view.

    - Other

      - Single Images - The IP camera shall support jpg file image screenshot and export.

      - Discovery - Manufacturer shall offer a discovery program to identify all devices of his manufacture on the network.

      - Emergency alarm – The IP camera shall allow sending of a video clip of configurable duration to up to five server locations.

      - Access - The IP camera shall permit up to five users to simultaneously access the camera.

  - NETWORK

    - Connectivity: 10/100 BASE-TX Ethernet with RJ-45 connector

    - Protocols supported

      - Transmission Control Protocol (TCP), Internet Protocol (IP) v4, User Datagram Protocol (UDP)

      - Configuration: Dynamic Host Configuration Protocol (DHCP)

      - Web services: Hypertext Transfer Protocol (HTTP), Secure Sockets Layer (SSL)

      - Network services: Domain Name System (DNS), Network Time Protocol (NTP), Internet Control Message Protocol (ICMP), Simple Network Management Protocol (SNMPv1/v2c/v3 MIB-2)

      - Media: Real-Time Transport Protocol (RTP), Real-Time Streaming Protocol (RTSP)

    - DDNS – The IP camera shall support DDNS services offered by the Manufacturer and other publicly available service offerings.

    - Security

      - The IP dome camera shall support IP address filtering whereby users can enter a list of allowed or blocked IP addresses for viewing video and configuring camera settings

      - The IP dome camera shall provide digest authentication, ID and password protection.

      - The IP dome camera shall provide three levels of user access with password protection.

  - ADDITIONAL FEATURES

    - System Information

      - The system settings of the IP camera shall be exportable as a separate file.

      - The IP camera shall maintain an accessible log of system and motion-triggered events.

        - The log shall be exportable to an Excel spreadsheet file.

  - CAMERA SOFTWARE

    - The IP camera shall have a built-in web server which supports browser-based configuration using Internet Explorer 8.0 or above

    - The software GUI shall allow access to camera information and all primary software functions, including:

      - Camera network address

      - Configuration

      - Stream control

      - Screen snapshot, print and export

      - Start and stop recording

    - The Manufacturer shall offer video viewer and configuration to implement the following actions:

      - Camera discovery

      - Configuration

        - video parameters

        - events and notifications

          - motion related

          - emergency

        - camera network parameters

        - SD card storage recording management

        - image capture, export, and print

      - Viewer - view video streams through the web browser

      - Image print and export

      - Instant record and playback

      - Alerts

        - e-mail setup

        - define web addresses for notifications

      - System

        - firmware upgrade

        - reset to factory default

        - set date, time, and NTP server synchronization

        - user access control

        - view and export camera settings

        - view system logs

  - ELECTRICAL

    - Power

      - Sources

        - 12 VDC

        - PoE (IEEE 802.3af Class 3)

      - Power Consumption

        - DC12V:	 Max 7.0W

        - PoE: 	Max 7.8W

    - Connectors

      - Ethernet: 			RJ-45 connector

      - External power (12 VDC): 	2-conductor power to terminal block

  - MECHANICAL AND ENVIRONMENTAL

    - Housing Material: 		Die cast Aluminum, polycarbonate dome

    - Configuration: 		Dome

    - Environmental Rating: 	IP-66

    - Dimensions (D x H):  	4.9 in. x 1.88 in. (125 mm x 48 mm)

    - Temperature:

      - Operating: 		-4° F to 122° F (-20° C to 50° C)

    - Humidity:			0 - 90%, non-condensing

END OF SECTION

- EXECUTION

  - INSTALLERS

    - Contractor personnel shall comply with all applicable state and local licensing requirements.

  - PREPARATION

    - The network design and configuration shall be verified for compatibility and performance with the camera(s).

    - Network configuration shall be tested and qualified by the Contractor prior to camera installation.

  - INSTALLATION

    - Contractor shall follow all Manufacturer published installation procedures and guidelines.

    - Before permanent installation of the system, the system shall be factory tested in conditions simulating the final installed environment

      - A report indicating successful test results shall be produced.

  - STORAGE

    - The IP camera hardware shall be stored in an environment where temperature and humidity are in the range specified by the Manufacturer.

END OF SECTION